FLOOD SUSCEPTIBILITY MAP MAKING

INPUTS REQUIRED:

LULC – with & without *builtup*

DIVA GIS for River (Brahmaputra .shp)

Downloaded the River shp for distance from river https://www.hydrosheds.org/products/hydrorivers#downloads

Optional refinement (for more precision)

If your Asia rivers shapefile includes many tiny or irrelevant streams (like drainage lines that don’t exist in your DEM), you can:

- Open Attribute Table → Select by Length (e.g., select streams longer than 1 km).

- Export selection as a new shapefile.

- Use that for Euclidean Distance.
→ This filters out noise while keeping all main and secondary channels.

  Source: https://www.hydrosheds.org/products/hydrorivers

  Coverage: Global, includes the Brahmaputra and all its tributaries.

  Format: Shapefile (.shp)

  Year: Based on SRTM data (~2000), but widely used for hydrological basins and river flow network.

  How to Use:

- Download HydroRIVERS Asia shapefile.

- Clip it using your AOI boundary (Kaziranga / Assam) in ArcMap or QGIS.

- Optionally, filter by MAIN_RIV = "Brahmaputra".

FOR RAINFALL DATA IMD NETCDF FILE DOWNLOADED FOR YEAR 2017

GEE RAINFALL MAP FROM CHIRPS

// --------------------------

// 1. Define your AOI

// Replace with your uploaded AOI asset

var aoi = ee.FeatureCollection('users/yourusername/aoi');

// --------------------------

// 2. Load CHIRPS daily rainfall for 2017

var dailyRain = ee.ImageCollection("UCSB-CHG/CHIRPS/DAILY")

.filterDate('2017-01-01', '2017-12-31');

// --------------------------

// 3. Aggregate daily rainfall to annual rainfall

var annualRain = dailyRain.reduce(ee.Reducer.sum());

// --------------------------

// 4. Clip annual rainfall to AOI

var annualRainClipped = annualRain.clip(aoi);

// --------------------------

// 5. Visualize on map

Map.centerObject(aoi, 9);

Map.addLayer(annualRainClipped, {

min: 0,

max: 4000,                // adjust according to expected rainfall

palette: ['lightblue','blue','darkblue']

}, 'Annual Rainfall 2017');

// --------------------------

// 6. Export the annual rainfall map as GeoTIFF

Export.image.toDrive({

image: annualRainClipped,

description: 'AnnualRainfall2017',

scale: 5000,              // adjust for resolution (5km ~ CHIRPS)

region: aoi.geometry(),

fileFormat: 'GeoTIFF',

maxPixels: 1e13

});

Weighted Overlay INFLUENCE